Power Systems Lab — Manual Calculations (EXP 1–15)

EXP-1: Load Curve & Load Duration Curve

Given Data:

Hourly Load Data (kW): 30,35,32,40,50,65,80,90,85,75,70,65,60,65,70,85,100,110,95,80,70,55,45,35

Connected Load = 150 kW

Formula Used:

Max Demand = highest value in load data

Average Load = Σ(load)/24

Load Factor = Average Load / Max Demand

Demand Factor = Max Demand / Connected Load

Calculation:

Max Demand = 110 kW

Average Load = Σ(load)/24 = 1587/24 = 66.12 kW

Load Factor = 66.12/110 = 0.6011

Demand Factor = 110/150 = 0.7333

Result:

Max Demand = 110.00 kW

Average Load = 66.12 kW

Load Factor = 0.6011 (60.11%)

Demand Factor = 0.7333 (73.33%)

EXP-2: Load Forecasting using Linear Regression

Given Data:

Years: 2018–2024

Load (MW): 120,128,135,145,150,162,170

Future Years: 2025,2026,2027,2028,2030

Formula Used:

Load = m×Year + c   (from polyfit, degree 1)

Calculation:

m = 8.29, c = -16607.14 (from regression)

Load(2025) = 8.29×2025 - 16607.14 = 177.57

Load(2026) = 8.29×2026 - 16607.14 = 185.89

Load(2027) = 194.21

Load(2028) = 202.54

Load(2030) = 219.18

Result:

2025: 177.57 MW | 2026: 185.89 MW | 2027: 194.21 MW

2028: 202.54 MW | 2030: 219.18 MW

EXP-3: Single-Area LFC Frequency Response

Given Data:

R = 2.4, D = 0.8, H = 5, f0 = 50 Hz

Tg = 0.08, Tt = 0.3

PL1 = 0.01, PL2 = 0.02, PL3 = 0.05 pu

Formula Used:

Tp = 2H/(D×f0)

Kp = 1/D

Δf(ss) = -ΔPL/(D + 1/R)

Calculation:

Tp = 2(5)/(0.8×50) = 0.25

Kp = 1/0.8 = 1.25

D + 1/R = 0.8 + 1/2.4 = 1.2167

Δf1 = -0.01/1.2167 = -0.0082 Hz

Δf2 = -0.02/1.2167 = -0.0164 Hz

Δf3 = -0.05/1.2167 = -0.0411 Hz

Result:

Δf(0.01pu) = -0.0082 Hz

Δf(0.02pu) = -0.0164 Hz

Δf(0.05pu) = -0.0411 Hz

EXP-4: Governor Speed-Load (Droop) Characteristic

Given Data:

R = 0.05 (5% droop)

Tg = 0.2 s

Formula Used:

ΔP = -(1/R) × Δf

Calculation:

Δf = -0.01 → ΔP = -(1/0.05)×(-0.01) = 0.2

Δf = -0.02 → ΔP = -(1/0.05)×(-0.02) = 0.4

Δf = -0.05 → ΔP = -(1/0.05)×(-0.05) = 1.0

Result:

Δf=-0.01 → ΔP=0.2 pu

Δf=-0.02 → ΔP=0.4 pu

Δf=-0.05 → ΔP=1.0 pu

EXP-5: Automatic Generation Control (AGC) — Single Area

Given Data:

R = 2.4, D = 0.8, H = 5, f0 = 50

Tg = 0.08, Tt = 0.3

PL = 0.02 pu

Formula Used:

Tp = 2H/(D×f0)

Kp = 1/D

Δf(ss) = -ΔPL/(D + 1/R)

Calculation:

Tp = 2(5)/(0.8×50) = 0.25

Kp = 1/0.8 = 1.25

D + 1/R = 0.8 + 0.4167 = 1.2167

Δf(ss) = -0.02/1.2167 = -0.01644 Hz

Result:

Steady-State Frequency Deviation = -0.01644 Hz

EXP-6: Two-Area Interconnected LFC

Given Data:

R1=R2 = 2.4, D1=D2 = 0.8

T12 = 0.545 (tie-line coefficient)

PL = 0.01 pu (applied to Area-1)

Formula Used:

Δf(ss) = -ΔPL/(D + 1/R)

ΔPtie(ss) = T12 × (Δf1 - Δf2)

Calculation:

D+1/R = 0.8+0.4167 = 1.2167

Δf1 = -0.01/1.2167 = -0.00822 Hz

Δf2 = -(0.5×0.01)/1.2167 = -0.00411 Hz

ΔPtie = 0.545 × (-0.00822 - (-0.00411))

= 0.545 × (-0.00411)

= -0.00224 pu

Result:

Δf1 = -0.00822 Hz

Δf2 = -0.00411 Hz

ΔPtie = -0.00224 pu

EXP-7: Automatic Voltage Regulator (AVR)

Given Data:

Ka=10, Ke=1, Kg=1, Ks=1

Formula Used:

Vt(ss) = (Ka×Ke×Kg) / (1 + Ka×Ke×Kg×Ks)

Error = 1 - Vt(ss)

Calculation:

Vt(ss) = (10×1×1)/(1+10×1×1×1) = 10/11 = 0.909 pu

Error = 1 - 0.909 = 0.091 → 9.1%

Result:

Steady-State Terminal Voltage = 0.909 pu

Steady-State Error = 9.1%

EXP-8: Shunt Capacitor — Power Factor Correction

Given Data:

VLL = 400 V, P = 10 kW, QL = 8 kVAR

PFtarget = 0.95, QC selected = 5 kVAR

Formula Used:

S1 = √(P²+QL²), PF1 = P/S1, I1 = S1/(√3×VLL)

Qtarget = P×tan(cos⁻¹(PFtarget)), QC_required = QL-Qtarget

Qnew = QL-QC, S2 = √(P²+Qnew²), PF2 = P/S2, I2 = S2/(√3×VLL)

Calculation:

S1 = √(10000²+8000²) = 12806 VA

PF1 = 10000/12806 = 0.781

I1 = 12806/(√3×400) = 18.48 A

Qtarget = 10000×tan(cos⁻¹0.95) = 3287 VAR

QC_required = 8000-3287 = 4713 VAR = 4.71 kVAR

Qnew = 8000-5000 = 3000 VAR

S2 = √(10000²+3000²) = 10440 VA

PF2 = 10000/10440 = 0.958

I2 = 10440/(√3×400) = 15.07 A

% Current Reduction = (18.48-15.07)/18.48×100 = 18.46%

Result:

Initial PF = 0.781, Initial Current = 18.48 A

Required Capacitor = 4.71 kVAR, Selected = 5 kVAR

Final PF = 0.958, Final Current = 15.07 A

Current Reduction = 18.46%

EXP-9: STATCOM/SVC Voltage Regulation

Given Data:

Base Voltage = 500 kV

Vref = 1.0 pu

Source Voltage Variation = 0.95 - 1.05 pu

STATCOM Rating = ±100 MVAR

Formula Used:

Actual Voltage = pu value × Base Voltage

Calculation:

Vref(actual) = 1.0×500 = 500 kV

Vmin(actual) = 0.95×500 = 475 kV

Vmax(actual) = 1.05×500 = 525 kV

Result:

Vref = 500 kV, Vmin = 475 kV, Vmax = 525 kV

QSTATCOM range = -100 MVAR to +100 MVAR

EXP-10: WLS State Estimation

Given Data:

z = [0.51; 0.18; 0.315]

H = [5 0; 0 2.5; -4 4]

σ² = 0.0001 → R = diag(0.0001,0.0001,0.0001)

Formula Used:

x_est = (Hᵀ R⁻¹ H)⁻¹ Hᵀ R⁻¹ z

Calculation:

Since R = σ²I, this simplifies to:

x_est = (HᵀH)⁻¹ Hᵀz

HᵀH = [41 -16; -16 22.25]

Hᵀz = [1.29; 1.71]

Solving: x_est = (HᵀH)⁻¹(Hᵀz)

Result:

θ2 = 0.0854 rad

θ3 = 0.1383 rad

EXP-11: SCADA/RTU Monitoring

Given Data:

Bus Voltage = 11 kV, Load = 50 MW, Breaker Status = Closed

Formula Used:

No formula — logic-based monitoring/control (if-else decision structure)

Calculation:

MTU Command = TRIP (0):

Breaker Status → OPEN

Load → 0 MW

Bus Voltage → 0 kV

MTU Command = CLOSE (1):

Breaker Status → CLOSED

Load → 50 MW

Bus Voltage → 11 kV

Result:

RTU successfully updates and transmits breaker status, load, and voltage

to Master Station based on MTU command.

EXP-12: Chi-Square Bad Data Detection

Given Data:

m = 5 (measurements), n = 3 (states)

J(x) = 12.5 (calculated)

χ² threshold (dof=2, 95% confidence) = 5.99

Formula Used:

dof = m - n

If J(x) > χ² threshold → Bad Data Detected

Calculation:

dof = 5 - 3 = 2

Compare: J(x) = 12.5  vs  χ² = 5.99

Since 12.5 > 5.99

Result:

Bad Data is DETECTED (J(x) exceeds chi-square threshold)

EXP-13: Economic Dispatch

Given Data:

PD = 300 MW

Unit 1: a1=500, b1=5, c1=0.01

Unit 2: a2=400, b2=6, c2=0.015

Formula Used:

P1 = (b2+2c2·PD-b1)/(2c1+2c2)

P2 = PD - P1

IC = b + 2c·P

F = a + bP + cP²

Calculation:

P1 = (6+2(0.015)(300)-5)/(2(0.01)+2(0.015)) = (6+9-5)/0.05 = 10/0.05 = 200 MW

P2 = 300-200 = 100 MW

IC1 = 5+2(0.01)(200) = 9

IC2 = 6+2(0.015)(100) = 9  (equal → confirms optimum)

F1 = 500+5(200)+0.01(200)² = 1900

F2 = 400+6(100)+0.015(100)² = 1150

Total Cost = 1900+1150 = 3050

Result:

P1 = 200 MW, P2 = 100 MW

IC1 = IC2 = 9 (verified optimal)

Total Generation Cost = 3050 cost/hour

EXP-14: Unit Commitment

Given Data:

Unit 1: Pmax=400 MW, Pmin=100 MW

Unit 2: Pmax=300 MW, Pmin=50 MW

Startup Cost: U1=500, U2=300

Hour 1 Load = 150 MW, Hour 2 Load = 450 MW

Formula Used:

Feasibility Check: Pmin ≤ Load ≤ Pmax

Total Cost = Running Cost + Startup Cost

Calculation:

HOUR 1 (Load=150 MW):

Unit1 alone: 100≤150≤400 → Feasible

Unit2 alone: 50≤150≤300 → Feasible

→ Choice exists between U1 or U2 alone

If Unit2 alone: Cost2 = 0.02(150)²+15(150)+150 = 2850

Total = 2850+300(startup) = 3150

HOUR 2 (Load=450 MW):

Unit1 alone: Pmax=400 < 450 → NOT sufficient

Unit2 alone: Pmax=300 < 450 → NOT sufficient

→ Both units must be ON

Total Capacity = 400+300 = 700 ≥ 450 → Feasible

Result:

Hour 1: Unit 2 alone sufficient, Total Cost = 3150

Hour 2: Both units required, Combined Capacity = 700 MW ≥ 450 MW

EXP-15: Demand Side Management — Load Shifting

Given Data:

Peak Demand = 1200 MW, Minimum Demand = 400 MW

Load Shifted = 150 MW

Peak Cost = ₹7429.20/MWh, Off-Peak Cost = ₹3714.60/MWh

Formula Used:

New Peak = Old Peak - Shift

New Valley = Old Valley + Shift

Average Load = (Peak+Valley)/2

Load Factor = Average Load/Peak Load

Savings = Shift × (Peak Cost - Off-Peak Cost)

Calculation:

New Peak = 1200-150 = 1050 MW

New Valley = 400+150 = 550 MW

Average Load = (1200+400)/2 = 800 MW

Old LF = 800/1200 = 0.667

New LF = 800/1050 = 0.762

Cost Saving/MWh = 7429.20-3714.60 = 3714.60

Total Savings = 150×3714.60 = ₹5,57,190/day

Result:

New Peak = 1050 MW, New Valley = 550 MW

Old LF = 0.667, New LF = 0.762

Daily Savings = ₹5,57,190

All 15 experiments complete — paste directly into your record. Good luck! 🍀